In AS/400 we have a Tape Library called TAPMLB02 which is having 10 tape drives, all are located in Frame 5.

1.Connect to the web console interface via link: http://ctclt28691

Welcome page will be displayed

2.Select Manage Drives to open sub-menu

3.Select Drive Summary to view drive info

4.Use Scroll bar to right to display requested drive

5.Click on Select box for requested drive, select one drive at a time

AS/400 drives are in Frame 5. Ensure that the correct drive is selected, and the drive must be empty.

6.Use option bar to select Clean

7.Press Go

A message stating " IBM does not recommend cleaning a drive manually" is displayed.

8.Select Clean option and the drive cleaning is complete

9.Repeat as necessary for all the 10 tape drives in Frame 5