Steps to be followed to perform Multi Path resetting for SAN-Disk(External storage) Connections using AS/400 Service Tool(SST)

Scenario Demands is while you receive CPI096E - Disk unit connection missing message.

- On the operating system command line, run the STRSST command.

- Select Option 1, Start a service tool, and press the Enter key.

- Select Option 4, Display/Alter/Dump, and press the Enter key.

- Select Option 1, Display/Alter storage, and press the Enter key.

- Select Option 2, Licensed Internal Code (LIC) data, and press the Enter key.

- Select Option 14, Advanced analysis (Page down to see this option), and press the Enter key.

- At the Select Advanced Analysis Command screen, scroll down, type 1 by MULTIPATHRESETTER, and press the Enter key.

- At the Specify Advanced Analysis Options screen, type -RESETMP-ALL and press the Enter key

- Read and follow the instructions displayed on the screen to confirm that the paths should be reset.