Login to the system using profile similar to QSECOFR and perform the below steps.

Step 1:
Check the status of the attached hardware resources whether they are operational or not. Using Command, WRKHDWRSC *STG or DSPHDWRSC.

Step 2:
Note down the controller resource name and model details of the resources.

Step 3:
Login to SST. Using command STRSST.

Step 4:
Take option 1. Start a service tool in System Service Tools (SST) screen.

Step 5:
Take option 7. Hardware service manager in Start a Service Tool screen.

Step 6:
Take option 2. Logical hardware resources (buses, IOPs, controllers) in Hardware Service Manager Screen.

Step 7:
Take option 1. System bus resources in Logical Hardware Resources screen.

Step 8:
Select Subset by *STG.

Step 9:
Locate the controller by resource name which we have already noted.

Step 10:
Take option 6=I/O debug in Logical Hardware Resources Associated with IOP screen.

Step 11:
Take option 3=Reset I/O processor and hit enter.
You will receive a message at the bottom saying Reset has been completed.

Step 12:
Take option 4. IPL I/O processor in Select IOP Debug Function screen.
You will receive a message at the bottom saying IPL has been completed.