Instruction to check life time of the cache battery using Service Tool (SST)

There are 2 ways to check the life time of the cache battery using service tool (SST).

Step 1:

Login to the system using profile having special authority *SERVICE

In the command line issue the command STRSST. Provide the SST ID and password.

Then take OPTION 1 = Start a service tool

OPTION 7 = Hardware service manager

OPTION 9 = Work with resources containing cache battery packs

Option 5 = Display battery information

Step 2:

In the command line issue the command STRSST. Provide the SST ID and password.

Then take OPTION 1 = Start a service tool

OPTION 4 = Display/Alter/Dump

OPTION 1 = Display/Alter storage

OPTION 2 = Licensed Internal Code (LIC) data

OPTION 14 = Advanced analysis

Take 1 against BATTERYINFO

In the options type -INFO -ALL